Lab7A: Configuring QoS on Cisco Routers

Objectives:

1. Create and apply QoS policies on Cisco routers.

2. Shape and police traffic across a WAN link.

3. Ensure preferential treatment for specific traffic types.

Topology:

RTR1 Configuration:

Int G0/0 = 200.100.10.1 /24

Int G0/1 = 192.168.11.1 /24

Int S0/0/0 = 10.10.10.1 /30 (512 kbps HDLC WAN link, DCE)

RTR2 Configuration:

Int G0/0 = 192.168.20.1 /24

Int G0/1 = 192.168.21.1 /24

Int S0/0/0 = 10.10.10.2 /30 (512 kbps HDLC WAN link)

Step 1: Configure Basic Settings

RTR1 Configuration:

enable

configure terminal

hostname RTR1

interface g0/0

ip address 200.100.10.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 192.168.11.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 10.10.10.1 255.255.255.252

clock rate 1000000

no shutdown

exit

ip route 0.0.0.0 0.0.0.0 10.10.10.2

RTR2 Configuration:

enable

configure terminal

hostname RTR2

interface g0/0

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 192.168.21.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 10.10.10.2 255.255.255.252

no shutdown

exit

ip route 0.0.0.0 0.0.0.0 10.10.10.1

Some traffic notes or wishes:

Important Data is traffic moving between 200.100.10.0 and 192.168.20.0 networks. I want this treated with some higher importance.

Bulk Data is just regular traffic that is moving between the 192.168.11.0 and 192.168.21.0 networks. I don’t need to be crazy strict with this traffic, but I would like some QOS applied.

TFTP traffic – this should be limited to some amount of bandwidth when things get busy.

Telnet traffic – for this lab, I want this given preference over other traffic. This would never be done in the real world.

Step 2: Define Class Maps

RTR1 and RTR2 Configurations (pay attention to which you are, as the ACLs will be mirrored):

class-map TFTP

match protocol tftp

class-map match-all IMPORTANTDATA

match access-group 101

class-map match-all TELNET

match protocol telnet

class-map match-all BULKDATA

match access-group 102

exit

ON RTR1:

access-list 101 permit ip 200.100.10.0 0.0.0.255 192.168.20.0 0.0.0.255

access-list 102 permit ip 192.168.11.0 0.0.0.255 192.168.21.0 0.0.0.255

RTR2 mirrors RTR1

Step 3: Define Policy Maps

RTR1 and RTR2 Configurations:

policy-map LAB7A

class TFTP

police 128000 conform-action transmit exceed-action drop

class IMPORTANTDATA

bandwidth 256

class TELNET

priority

class BULKDATA

shape average 128000

class class-default

fair-queue

Step 4: Apply Service Policy

RTR1 and RTR2 Configuration:

interface s0/0/0

service-policy output LAB7A

Now you have QOS applied to an interface. You can see this by using the command

show policy-map interface s0/0/0 and paste the output here:

End of Lab Questions:

1. What is the purpose of class maps in QoS configuration?

Class maps are part of QoS configuration for classification of traffic into different categories.

2. How does the `police` command differ from the `shape` command in QoS configuration?

Police forces traffic rate limits and drops excess traffic. While shape smooths traffic to a specified rate without dropping excess.

3. What is the function of the `priority` keyword in a policy map?

The priority keyword ensures that traffic in the defined class is considered high priority and low delay. This is usually used for latency sensitive traffic like voice traffic.

4. Why is it important to apply a service policy to an interface?

Applying a service policy to an interface is important to force QoS settings. For example, defining priority, shaping or policy on the traffic going through that interface.

When finished, please submit as yourname_Lab7a.doc