Module 8 Lab – Site-to-Site VPN

Topology

IP Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A | Port 1 |
|  | G0/0 | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
| R2 | G0/1 | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
|  | G0/2 | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| R3 | G0/1 | 192.168.3.1 | 255.255.255.0 | N/A | Port 1 |
|  | G0/0 | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | Port 2 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 | Port 2 |

Objectives

Part 1: Configure Basic Device Settings

Configure hostnames, interface IP addresses, and access passwords.

Configure the OSPF dynamic routing protocol.

Part 2: Configure a Site-to-Site VPN Using Cisco IOS

Configure IPsec VPN settings on R1 and R3.

Verify site-to-site IPsec VPN configuration.

Test IPsec VPN operation.

Background / Scenario

VPNs can provide a secure method of transmitting data over a public network, such as the Internet. VPN connections can help reduce the costs associated with leased lines. Site-to-Site VPNs typically provide a secure (IPsec or other) tunnel between a branch office and a central office. Another common implementation of VPN technology is remote access to a corporate office from a telecommuter location, such as a small office or home office.

In this lab, you will build and configure a multi-router network, use Cisco IOS to configure a site-to-site IPsec VPN, and then test the VPN. The IPsec VPN tunnel is from R1 to R3 via R2. R2 acts as a pass-through and has no knowledge of the VPN. IPsec provides secure transmission of sensitive information over unprotected networks, such as the Internet. IPsec acts at the network layer and protecting and authenticating IP packets between participating IPsec devices (peers), such as Cisco routers.

Required Resources

3 routers (Cisco 1941 with Cisco IOS Release 15.4(3)M2 image with a Security Technology package license)

2 switches (Cisco 2960 or comparable) (not required)

2 PCs

Ethernet cables, as shown in the topology and console cables to configure Cisco networking devices.

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, dynamic routing, device access, and passwords.

Note: All tasks should be performed on R1, R2, and R3. The procedure for R1 is shown here as an example.

Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram and cable as necessary.

Configure basic settings for each router.

Configure hostnames, as shown in the topology.

Configure the interface IP addresses, as shown in the IP Addressing Table.

Disable DNS lookup.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands.

Conf t

No ip domain-lookup

Configure the OSPF routing protocol on R1, R2, and R3.

On R1, use the following commands:

R1(config)# router ospf 101

R1(config-router)# network 192.168.1.0 0.0.0.255 area 0

R1(config-router)# network 10.1.1.0 0.0.0.3 area 0

On R2, use the following commands:

R2(config)# router ospf 101

R2(config-router)# network 10.1.1.0 0.0.0.3 area 0

R2(config-router)# network 10.2.2.0 0.0.0.3 area 0

On R3, use the following commands:

R3(config)# router ospf 101

R3(config-router)# network 192.168.3.0 0.0.0.255 area 0

R3(config-router)# network 10.2.2.0 0.0.0.3 area 0

Configure PC host IP settings.

Configure a static IP address, subnet mask, and default gateway for PC-A, as shown in the IP Addressing Table.

Configure a static IP address, subnet mask, and default gateway for PC-C, as shown in the IP Addressing Table.

Verify basic network connectivity.

Ping from R1 to the R3 Fa0/1 interface at IP address 192.168.3.1.

If the pings are unsuccessful, troubleshoot the basic device configurations before continuing.

Ping from PC-A on the R1 LAN to PC-C on the R3 LAN.

If the pings are unsuccessful, troubleshoot the basic device configurations before continuing.

Note: If you can ping from PC-A to PC-C, you have demonstrated that the OSPF routing protocol is configured and functioning correctly. If you cannot ping, but the device interfaces are up and IP addresses are correct, use the show run and show ip route commands to help identify routing protocol-related problems.

    - Configure passwords so that you can telnet

Configure the same settings for R1 and R3. R1 is shown here as an example.

- Conf t

Enable secret Pass-12345

Line vty 0 4

Password Pass-12345

Transport input all

telnet working

Configure a Site-to-Site VPN with Cisco IOS

In Part 2 of this lab, you will configure an IPsec VPN tunnel between R1 and R3 that passes through R2. You will configure R1 and R3 using the Cisco IOS CLI. You will then review and test the resulting configuration.

Configure IPsec VPN Settings on R1 and R3.

Verify connectivity from the R1 LAN to the R3 LAN.

In this task, you will verify that PC-A on the R1 LAN can ping PC-C on the R3 LAN with no tunnel in place.

Ping the PC-C IP address of 192.168.3.3 from PC-A.

PC-A:\> ping 192.168.3.3

If the pings are unsuccessful, troubleshoot the basic device configurations before continuing.

Enable IKE policies on R1 and R3.

IPsec is an open framework that allows for the exchange of security protocols as new technologies, and encryption algorithms as they are developed.

There are two central configuration elements in the implementation of an IPsec VPN:

Implement Internet Key Exchange (IKE) parameters

Implement IPsec parameters

Verify that IKE is supported and enabled.

IKE Phase 1 defines the key exchange method used to pass and validate IKE policies between peers. In IKE Phase 2, the peers exchange and match IPsec policies for the authentication and encryption of data traffic.

IKE must be enabled for IPsec to function. IKE is enabled, by default, on IOS images with cryptographic feature sets. If it is disabled, you can enable it with the crypto isakmp enable command. Use this command to verify that the router IOS supports IKE and that it is enabled.

R1(config)# crypto isakmp enable

R3(config)# crypto isakmp enable

Note: If you cannot execute this command on the router, you must upgrade to the IOS image that includes the Cisco cryptographic services.  For Cisco 2911’s run the following commands:

Conf t

License boot module c2900 technology-package securityK9

The router will then ask you to accept, select Yes.

You will also need to run the command “do write”.  Then Ctrl-Z, then “wr” or copy run start.

Establish an ISAKMP policy and view the available options.

To allow IKE Phase 1 negotiation, you must create an ISAKMP policy and configure a peer association involving that ISAKMP policy. An ISAKMP policy defines the authentication and encryption algorithms and the hash function used to send control traffic between the two VPN endpoints. When an ISAKMP security association has been accepted by the IKE peers, IKE Phase 1 has been completed. IKE Phase 2 parameters will be configured later.

Issue the crypto isakmp policy number global configuration mode command on R1 for policy 10.

R1(config)# crypto isakmp policy 10

View the various IKE parameters available using Cisco IOS help by typing a question mark (?).

R1(config-isakmp)# ?

ISAKMP commands:

authentication  Set authentication method for protection suite

default         Set a command to its defaults

encryption      Set encryption algorithm for protection suite

exit            Exit from ISAKMP protection suite configuration mode

group           Set the Diffie-Hellman group

hash            Set hash algorithm for protection suite

lifetime        Set lifetime for ISAKMP security association

no              Negate a command or set its defaults

Configure the IKE Phase 1 ISAKMP policy on R1 and R3.

Your choice of an encryption algorithm determines how confidential the control channel between the endpoints is. The hash algorithm controls data integrity, ensuring that the data received from a peer has not been tampered with in transit. The authentication type ensures that the packet was sent and signed by the remote peer. The Diffie-Hellman group is used to create a secret key shared by the peers that has not been sent across the network.

Configure an ISAKMP policy with a priority of 10. Use pre-shared key as the authentication type, aes 256 for the encryption algorithm, sha as the hash algorithm, and the Diffie-Hellman group 14 key exchange. Give the policy a lifetime of 3600 seconds (one hour).

Note: Older versions of Cisco IOS do not support AES 256 encryption and SHA as a hash algorithm. Substitute whatever encryption and hashing algorithm your router supports. Ensure that the same changes are made on R3 in order to be in sync.

R1(config)# crypto isakmp policy 10

R1(config-isakmp)# hash sha

R1(config-isakmp)# authentication pre-share

R1(config-isakmp)# group 14

R1(config-isakmp)# lifetime 3600

R1(config-isakmp)# encryption aes 256

R1(config-isakmp)# end

Configure the same policy on R3.

R3(config)# crypto isakmp policy 10

R3(config-isakmp)# hash sha

R3(config-isakmp)# authentication pre-share

R3(config-isakmp)# group 14

R3(config-isakmp)# lifetime 3600

R3(config-isakmp)# encryption aes 256

R3(config-isakmp)# end

Verify the IKE policy with the show crypto isakmp policy command.

R1# show crypto isakmp policy

Global IKE policy

Protection suite of priority 10

encryption algorithm:   AES - Advanced Encryption Standard (256 bit keys).

hash algorithm:         Secure Hash Standard

authentication method:  Pre-Shared Key

Diffie-Hellman group:   #14 (2048 bit)

lifetime:               3600 seconds, no volume limit

Configure pre-shared keys.

Because pre-shared keys are used as the authentication method in the IKE policy, a key must be configured on each router that points to the other VPN endpoint. These keys must match for authentication to be successful. The global configuration mode crypto isakmp key <key-string> address <ip-address> command is used to enter a pre-shared key. Use the IP address of the remote peer, which is the remote interface that the peer would use to route traffic to the local router.

Which IP addresses should you use to configure the IKE peers, given the topology diagram and IP addressing table?

G0/0 on Router 1 – ip address 10.1.1.1

G0/0 on Router 3 – ip address 10.2.2.1

Each IP address that is used to configure the IKE peers is also referred to as the IP address of the remote VPN endpoint. Configure the pre-shared key of cisco123 on router R1. Production networks should use a complex key. This command points to the remote peer R3 IP address.

R1(config)# crypto isakmp key cisco123 address 10.2.2.1

Configure the pre-shared key cisco123 on router R3. The command for R3 points to the R1 peer IP address.

R3(config)# crypto isakmp key cisco123 address 10.1.1.1

Configure the IPsec transform set and lifetime. (Let Phase II Begin)

The IPsec transform set is another crypto configuration parameter that routers negotiate to form a security association. To create an IPsec transform set, use the crypto ipsec transform-set <tag> command. Use ? to see which parameters are available.

R1(config)# crypto ipsec transform-set 50 ?

ah-md5-hmac   AH-HMAC-MD5 transform

ah-sha-hmac   AH-HMAC-SHA transform

comp-lzs      IP Compression using the LZS compression algorithm

esp-3des      ESP transform using 3DES(EDE) cipher (168 bits)

esp-aes       ESP transform using AES cipher

esp-des       ESP transform using DES cipher (56 bits)

esp-md5-hmac  ESP transform using HMAC-MD5 auth

esp-null      ESP transform w/o cipher

esp-seal      ESP transform using SEAL cipher (160 bits)

esp-sha-hmac  ESP transform using HMAC-SHA auth

On R1 and R3, create a transform set with tag 50 and use an ESP transform with an AES 256 cipher with ESP and the SHA hash function. The transform sets must match.

R1(config)# crypto ipsec transform-set 50 esp-aes 256 esp-sha-hmac

R1(cfg-crypto-trans)# exit

R3(config)# crypto ipsec transform-set 50 esp-aes 256 esp-sha-hmac

R3(cfg-crypto-trans)# exit

What is the function of the IPsec transform set?

To choose the same encryption method so both sides know how to decipher the message.

You can also change the IPsec security association lifetime from the default of 3600 seconds. On R1 and R3, set the IPsec security association lifetime to 30 minutes, or 1800 seconds.

R1(config)# crypto ipsec security-association lifetime seconds 1800

R3(config)# crypto ipsec security-association lifetime seconds 1800

Define interesting traffic with a CryptoACL.

To make use of the IPsec encryption with the VPN, it is necessary to define extended access lists to tell the router which traffic to encrypt. A packet that is permitted by an access list used for defining IPsec traffic is encrypted if the IPsec session is configured correctly. A packet that is denied by one of these access lists is not dropped it is sent unencrypted. Also, like any other access list, there is an implicit deny at the end, which means the default action is to not encrypt traffic. If there is no IPsec security association correctly configured, no traffic is encrypted and traffic is forwarded unencrypted.

In this scenario, from the perspective of R1, the traffic you want to encrypt is traffic going from R1’s Ethernet LAN to R3’s Ethernet LAN or vice versa from the perspective of R3. These access lists are used outbound on the VPN endpoint interfaces and must mirror each other.

Configure the IPsec VPN interesting traffic ACL on R1.

R1(config)# access-list 101 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255

Configure the IPsec VPN interesting traffic ACL on R3.

R3(config)# access-list 101 permit ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255

Does IPsec evaluate whether the access lists are mirrored as a requirement to negotiate its security association?

No. The access lists have to be configured besides the IPsec configuration.

Create and apply a crypto map.

A crypto map associates traffic that matches an access list to a peer and various IKE and IPsec settings. After the crypto map is created, it can be applied to one or more interfaces. The interfaces that it is applied to should be the ones facing the IPsec peer.

To create a crypto map, use crypto map <name> <sequence-num> <type> command in global configuration mode to enter crypto map configuration mode for that sequence number. Multiple crypto map statements can belong to the same crypto map and are evaluated in ascending numerical order. Enter crypto map configuration mode on R1. Use a type of ipsec-isakmp, which means IKE is used to establish IPsec security associations.

Create the crypto map on R1, name it CMAP, and use 10 as the sequence number. A message displays after the command is issued.

R1(config)# crypto map CMAP 10 ipsec-isakmp

% NOTE: This new crypto map will remain disabled until a peer

and a valid access list have been configured.

Use the match address <access-list> command to specify which access-list defines which traffic to encrypt.

R1(config-crypto-map)# match address 101

To view the list of possible set commands that you can do with a crypto map, use the help function.

R1(config-crypto-map)# set ?

identity              Identity restriction.

ip                    Interface Internet Protocol config commands

isakmp-profile        Specify isakmp Profile

nat                   Set NAT translation

peer                  Allowed Encryption/Decryption peer.

pfs                   Specify pfs settings

reverse-route         Reverse Route Injection.

security-association  Security association parameters

transform-set         Specify list of transform sets in priority order

Setting a peer IP or hostname is required. Set it to R3’s remote VPN endpoint interface using the following command.

R1(config-crypto-map)# set peer 10.2.2.1

Use the set transform-set <tag> command to hard code the transform set to be used with this peer. Set the perfect forwarding secrecy type using the set pfs <type> command, and modify the default IPsec security association life time with the set security-association lifetime seconds <seconds> command.

R1(config-crypto-map)# set pfs group14

R1(config-crypto-map)# set transform-set 50

R1(config-crypto-map)# set security-association lifetime seconds 900

R1(config-crypto-map)# exit

Create a mirrored matching crypto map on R3.

R3(config)# crypto map CMAP 10 ipsec-isakmp

R3(config-crypto-map)# match address 101

R3(config-crypto-map)# set peer 10.1.1.1

R3(config-crypto-map)# set pfs group14

R3(config-crypto-map)# set transform-set 50

R3(config-crypto-map)# set security-association lifetime seconds 900

R3(config-crypto-map)# exit

Apply the crypto map to interfaces.

Note: The SAs are not established until the crypto map has been activated by interesting traffic. The router generates a notification that crypto is now on.

Apply the crypto maps to the appropriate interfaces on R1 and R3.

R1(config)# interface G0/0

R1(config-if)# crypto map CMAP

*Jan 28 04:09:09.150: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON

R1(config)# end

R3(config)# interface G0/0

R3(config-if)# crypto map CMAP

*Jan 28 04:10:54.138: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON

R3(config)# end

Verify the Site-to-Site IPsec VPN Configuration.

    - Verify the IPsec configuration on R1 and R3.

Previously, you used the show crypto isakmp policy command to display the configured ISAKMP policies on the router. The show crypto ipsec transform-set command displays the configured IPsec policies in the form of the transform sets.

- Example:  (cut and paste yours here instead of this one)

R1# show crypto ipsec transform-set

R3# show crypto ipsec transform-set

Transform set 50: { esp-256-aes esp-sha-hmac  }

will negotiate = { Tunnel,  },

Transform set #$!default_transform_set_1: { esp-aes esp-sha-hmac  }

will negotiate = { Transport,  },

Transform set #$!default_transform_set_0: { esp-3des esp-sha-hmac  }

will negotiate = { Transport,  },

Use the show crypto map command to display the crypto maps that will be applied to the router.

- Example:  (cut and paste yours here instead of this one)

R1# show crypto map

Crypto Map "CMAP" 10 ipsec-isakmp

Peer = 10.2.2.1

Extended IP access list 101

access-list 101 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255

Current peer: 10.2.2.1

Security association lifetime: 4608000 kilobytes/900 seconds

Responder-Only (Y/N): N

PFS (Y/N): Y

DH group:  group14

Transform sets={

50:  { esp-256-aes esp-sha-hmac  } ,

}

Interfaces using crypto map CMAP:

Gigabit 0/0

R3# show crypto map

Crypto Map "CMAP" 10 ipsec-isakmp

Peer = 10.1.1.1

Extended IP access list 101

access-list 101 permit ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255

Current peer: 10.1.1.1

Security association lifetime: 4608000 kilobytes/900 seconds

Responder-Only (Y/N): N

PFS (Y/N): Y

DH group:  group14

Transform sets={

50:  { esp-256-aes esp-sha-hmac  } ,

}

Interfaces using crypto map CMAP:

Gigabit 0/0

Note: The output of these show commands does not change if interesting traffic goes across the connection. You test various types of traffic in the next task.

Verify the IPsec VPN Operation.

    - Display ISAKMP security associations.

The show crypto isakmp sa command reveals that no IKE SAs exist yet. When interesting traffic is sent, this command output will change.

R1# show crypto isakmp sa

IPv4 Crypto ISAKMP SA

dst             src             state          conn-id status

IPv6 Crypto ISAKMP SA

Display IPsec Security Associations (SA).

The show crypto ipsec sa command shows the unused SA between R1 and R3.

Note: The number of packets sent across is zero, and there is a lack of any security associations listed toward the bottom of the output. The output for R1 is shown here.

R1# show crypto ipsec sa

interface: Gigabit 0/0

Crypto map tag: CMAP, local addr 10.1.1.1

protected vrf: (none)

local  ident (addr/mask/prot/port): (192.168.1.0/255.255.255.0/0/0)

remote ident (addr/mask/prot/port): (192.168.3.0/255.255.255.0/0/0)

current_peer 10.2.2.1 port 500

PERMIT, flags={origin_is_acl,}

#pkts encaps: 0, #pkts encrypt: 0, #pkts digest: 0

#pkts decaps: 0, #pkts decrypt: 0, #pkts verify: 0

#pkts compressed: 0, #pkts decompressed: 0

#pkts not compressed: 0, #pkts compr. failed: 0

#pkts not decompressed: 0, #pkts decompress failed: 0

#send errors 0, #recv errors 0

local crypto endpt.: 10.1.1.1, remote crypto endpt.: 10.2.2.1

path mtu 1500, ip mtu 1500, ip mtu idb Gigabit 0/0

current outbound spi: 0x0(0)

PFS (Y/N): N, DH group: none

inbound esp sas:

inbound ah sas:

inbound pcp sas:

outbound esp sas:

outbound ah sas:

outbound pcp sas:

Why haven’t any SAs been negotiated?

Connection has not been established yet.

Generate some uninteresting test traffic and observe the results.

Ping from R1 to the R3 interface IP address 10.2.2.1. These pings should be successful.

Issue the show crypto isakmp sa command.

Ping from R1 to the R3 G0/1 interface IP address 192.168.3.1. These pings should be successful.

Issue the show crypto isakmp sa command again. Was an SA created for these pings? Explain.

No. I believe ping packets are not used for sa.

Issue the debug ip ospf hello command. You should see OSPF hello packets passing between R1 and R3.

R1# debug ip ospf hello

OSPF hello events debugging is on

R1#

*Apr  7 18:04:46.467: OSPF: Send hello to 224.0.0.5 area 0 on GigabitEthernet0/1 from 192.168.1.1

*Apr  7 18:04:50.055: OSPF: Send hello to 224.0.0.5 area 0 on Gigabit 0/0 from 10.1.1.1

*Apr  7 18:04:52.463: OSPF: Rcv hello from 10.2.2.2 area 0 from Gigabit 0/0 10.1.1.2

*Apr  7 18:04:52.463: OSPF: End of hello processing

*Apr  7 18:04:55.675: OSPF: Send hello to 224.0.0.5 area 0 on GigabitEthernet0/1 from 192.168.1.1

*Apr  7 18:04:59.387: OSPF: Send hello to 224.0.0.5 area 0 on Gigabit 0/0 from 10.1.1.1

*Apr  7 18:05:02.431: OSPF: Rcv hello from 10.2.2.2 area 0 from Gigabit 0/0 10.1.1.2

*Apr  7 18:05:02.431: OSPF: End of hello processing

Turn off debugging with the no debug ip ospf hello or undebug all command.

Re-issue the show crypto isakmp sa command. Was an SA created between R1 and R3? Explain.

No association because the traffic does not match the ACL created before. So it is not an interesting traffic still.

Generate some interesting test traffic and observe the results.

Use an extended ping from R1 to the R3 G0/1 interface IP address 192.168.3.1. Extended ping allows you to control the source address of the packets. Respond as shown in the following example. Press Enter to accept the defaults, except where a specific response is indicated.

R1# ping

Protocol [ip]:

Target IP address: 192.168.3.1

Repeat count [5]:

Datagram size [100]:

Timeout in seconds [2]:

Extended commands [n]: y

Source address or interface: 192.168.1.1

Type of service [0]:

Set DF bit in IP header? [no]:

Validate reply data? [no]:

Data pattern [0xABCD]:

Loose, Strict, Record, Timestamp, Verbose[none]:

Sweep range of sizes [n]:

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

..!!!

Success rate is 100 percent (3/5), round-trip min/avg/max = 92/92/92 ms

Re-issue the show crypto isakmp sa command.

R1# show crypto isakmp sa

IPv4 Crypto ISAKMP SA

dst             src             state          conn-id status

10.2.2.1        10.1.1.1        QM_IDLE           1001 ACTIVE

IPv6 Crypto ISAKMP SA

Why was an SA created between R1 and R3 this time?

Because now it is an interesting traffic that matches the ACL that was associated with the tunnel.

What are the endpoints of the IPsec VPN tunnel?

- The endpoints would be R1 g0/0 and R3 g0/0

Ping from PC-A to PC-C. If the pings were successful, issue the show crypto ipsec sa command. How many packets have been transformed between R1 and R3?

16 packets.

R1# show crypto ipsec sa

interface: Gigabit 0/0

Crypto map tag: CMAP, local addr 10.1.1.1

protected vrf: (none)

local  ident (addr/mask/prot/port): (192.168.1.0/255.255.255.0/0/0)

remote ident (addr/mask/prot/port): (192.168.3.0/255.255.255.0/0/0)

current_peer 10.2.2.1 port 500

PERMIT, flags={origin_is_acl,}

#pkts encaps: 7, #pkts encrypt: 7, #pkts digest: 7

#pkts decaps: 7, #pkts decrypt: 7, #pkts verify: 7

#pkts compressed: 0, #pkts decompressed: 0

#pkts not compressed: 0, #pkts compr. failed: 0

#pkts not decompressed: 0, #pkts decompress failed: 0

#send errors 2, #recv errors 0

local crypto endpt.: 10.1.1.1, remote crypto endpt.: 10.2.2.1

path mtu 1500, ip mtu 1500, ip mtu idb Gigabit 0/0

current outbound spi: 0xC1DD058(203280472)

inbound esp sas:

spi: 0xDF57120F(3747025423)

transform: esp-256-aes esp-sha-hmac ,

in use settings ={Tunnel, }

conn id: 2005, flow_id: FPGA:5, crypto map: CMAP

sa timing: remaining key lifetime (k/sec): (4485195/877)

IV size: 16 bytes

replay detection support: Y

Status: ACTIVE

inbound ah sas:

inbound pcp sas:

outbound esp sas:

spi: 0xC1DD058(203280472)

transform: esp-256-aes esp-sha-hmac ,

in use settings ={Tunnel, }

conn id: 2006, flow_id: FPGA:6, crypto map: CMAP

sa timing: remaining key lifetime (k/sec): (4485195/877)

IV size: 16 bytes

replay detection support: Y

Status: ACTIVE

outbound ah sas:

outbound pcp sas:

The previous example used pings to generate interesting traffic. What other types of traffic would result in an SA forming and tunnel establishment?

Any traffic that matched the extended ACL created. So any traffic from the internal network of PCA and from the internal network of PCC that are going of the their respective network.

Reflection

Would traffic on the Gigabit Ethernet link between PC-A and the R1 G0/0 interface be encrypted by the site-to-site IPsec VPN tunnel? Explain.

- No because it does not leave that the tunnel. So it would stay in the same network.