Applied Studies II

Logbook

Yuri Silva

Task: 3087 – Troubleshooting AP and network connectivity with AP

Location: E1019

Date: March 25th

Time: 09:30AM – 12:30AM

Time Billed = 3 hours @ $75.00/hr = $225.00

After Tyler configured the Access Point (192.168.143.8) there was no access from the network. For example, we were not able to ping the AP from the External Router, Internal router or any other device.

To troubleshoot first I checked if the 192.168.143.0 network was still working. So I ping different devices in the network and they were all successful. So there was no problem with the network itself.

Then I tested connecting my own computer to the same port on the wall the AP was connected to. In other words, I disconnected the AP and connected my computer to the wall. Then I configured a fixed IP address in the 143 network in my computer. I tested the connectivity with other devices (External router, DNS Server, Internal router) they were all successful.

So we realized there was no problem with the wall jack, cables or 143 network configuration.

We tried to reconfigure the AP from scratch and try one more time.

After all the configuration was done, we tested and the ping from the AP to other devices in the network were successful.

Task: 3088 – Trobleshooting Radius Server on DC1

Location: E1019

Date: March 26th

Time: 13:30AM – 15:30AM

Time Billed = 2 hours @ $75.00/hr = $150.00

After the AP was configured to connect to the radius server (shared key ILoveSurfing2!) it was still not working properly along with the Radius server on DC1.

I realized there was the Certification Authority was not properly configured on DC1. So I tried to add the template to issue a certificate for RAS and there was no templates showing up.

I needed to uninstall and install again the CA.

After that was completed, the templates were showing up and I was able to configure the CA to issue RAS and IAS Server.

When I tried to request a certificate from DC just for testing purposes it was still not showing up. So, it seems DC still cannot issue certificates for the radius server connection.

I am going to have to work more to figure out that the problem could be and how I could fix it.

Bill for this week.

Pictures:

Certification Authority configured to issue RAS and IAS Server for authentication of clients accessing the AP.

Network Policy Server configure to request Active directory authentication and Cisco AP Authentication.

NPS Configured with Wireless Access Policy and MS-CHAP v2 Policy.

AP configured as a RADIUS Clients with shared key ILoveSurfing2!