Applied Studies II

Logbook

Yuri Silva

Task: 3094 – Configuring Replacement Router

Location: E1019

Date: April 8th

Time: 9:00AM – 12:30AM

Time Billed = 3.5 hours @ $75.00/hr = $226.50

- The HSRP for the external router stopped working.

- When investigating we found out a problem with the flash memory. So the system would not boot the IOS.

- Consequently, it would not work as intended. It would stay only in the bootstrap screen.

- When talking to the team and the instructor, it was decided to replace the router.

- Another router was picked up at C2001 and brought to E1019.

- We connected the cables and started the configuration.

- We had a lot of issues when trying to load the backup configuration from the tftp server to the External Router 2.

- That was mainly because the backup configuration for the External Router 2 was missing on the tftp server.

- When decided to use the same configuration as the External Router 1.

- However, this generated other problems because when we loaded the configuration there was duplicate Ips and many log errors being generated because two routers had the same configuration.

- We couldn’t finish the configuration on this day and decided to continue on the following day.

- This task was performed along with Tyler.

Task: 3095 – Continuing the configuration of replacement router

Location: E1019

Date: April 9th

Time: 1:30PM – 3:30PM

Time Billed = 2 hours @ $75.00/hr = $150.00

- We had to use console cables to access both External routers because the Cyclades box was not working properly.

- This time, we decided to edit the backup configuration file before uploading it to External Router 2 (HSRP Router).

- We added the IP address, standby configuration, and all other parameters for the HSRP to work properly.

- Changed the router’s name as well so it would not conflict with External Router 1.

- Basically, we used the same file as External Router 1 but edited all the parameters to make it work as External Router 2.

- That backup file was edited from the TFTP Server (Linux VM).

- We used vim and changed all the parameters.

- After the new file was ready to be loaded, we copied the file from the tftp server to External Router 2.

- This time it worked flawlessly.

- We were able to have standby working properly.

- Showing one router as active and the other one as Standby.

Bill for this week.