Applied Studies II

Logbook

Yuri Silva

Task: 3019 – Creating automatic backup configuration for routers

Location: E1019

Date: January 28th

Time: 09:00AM – 10:30AM

Time Billed = 1.5 hours @ $75.00/hr = $112.50

- An event policy was created to run a command automatically at 2am every Wednesday.

- It is important to mention that a configuration for a quiet prompt needed to be added.

- Otherwise, when the copy running-config tftp command would run it would ask the confirmation from the user. With the quiet, it would not need additional confirmation. It would just copy the running config.

- This configuration added for both Internal and External routers.

Task: 3020 – Map all ports and devices through show cdp neighbors

Location: E1019

Date: January 29th

Time: 09:00AM – 10:30AM

Time Billed = 1.5 hours @ $75.00/hr = $112.50

- Ran show cdp neighbours on IntSW1, ExtRouter, ExtSW, IntSW2, IntSw3, IntRouter.

- Created a map with all ports and devices showing how each device was connected.

- Drawn a simple sketch to use as base for designing network diagrams.

Task: 3021 – Updated routing tables on External Router. Configured VLANs/trunk on Switches

Location: E1019

Date: January 29th

Time: 1:30PM – 3:00PM

Time Billed = 1.5 hours @ $75.00/hr = $112.50

- A new entry on the routing table of the external router was added so Windows and Linux boxes could have access to the network.

- Also the Vlans were assigned to the specific port according to the instructions and cabling on the corresponding Switch.

- Trunk ports were configured according to the specifications as well.

Bill for this week.

Pictures:

The Backup config files were moved to a backup folder according to the instructions.

Sketch of network infrastructure

Ip route on the External router updated.

Vlans configured on the Internal Switch 1

Vlans configured on IntSW2

Vlans on IntSW3

Trunk configured on IntSW1

Trunk ports configured on IntSW2

Trunk configuration on IntSW3