Applied Studies II

Logbook

Yuri Silva

Task: 3044 – Reconfigure ACLs on Internal Router

Location: E1019

Date: February 11th

Time: 10:00AM – 11:30AM

Time Billed = 1 hours @ $75.00/hr = $75.00

This task was done because the initial configuration of the ACLs was not correct.

So, in order to block traffic from Team 2 and Team 4 a new ACLs were created and added to the interface g0/2 of the Internal router (inbound traffic).

The ACLs concept is to block access from Networks:

Deny 192.168.142.0/24

Deny 192.168.144.0/24

Deny 192.168.152.0/24

Deny 192.168.154.0/24

Permit any

Task: 3045 – Configure NAT/PAT on External Router

Date: February 12th

Time: 11:30AM – 1:00PM

Time Billed = 1.5 hours @ $75.00/hr = $75.00

NAT/PAT was configured on the External router so users from the 192.168.143.0/24 network would be able to access the Internet.

The internal interface for NAT is g0/1 where the .153 network is located. This Interface is connected to the internal router.

The external interface for NAT g/0/0 is where the .207 network is located. This Interface will point to the school’s network for the outside access.

Task: 3046 – Configure NTP on internal router / Review automatic backup configuration / Review routing table

Location: E1019

Date: February 12th

Time: 13:30AM – 15:00AM

Time Billed = 1.5 hours @ $75.00/hr = $75.00

Regarding NTP on the internal router:

- The NTP was missing on the internal router. So, I added the command to use the External router as the reference for the time. Since the External router is the NTP server.

- I also updated the time zone to follow UTC -4 which is the time zone of Atlantic Canada.

For the Automatic backup configuration.

For test purposes I updated the command to backup the configuration of the External Router to the ftp server every hour.

The result is that the configuration is being saved automatically. The picture I added to this document shows the configuration being saved at 10:59AM of today (last hour). The time now is 11:50AM.

The next step now is to add the configuration for the Internal router and add the proper time to execute the command according to the specifications (once a week).

New entries on the routing table

Both external and internal routers now have the default route pointing to the next hop IP address instead of pointing to the next interface.

The internal router has now entries for networks:

.141, .145, .151, .155

Bill for this week.

Pictures:

NAT created in the External router.

Routes were added on the External router.

Routes updated on the Internal router

NTP server set up in the internal router.

ACLs created in the internal router.

Added to int g0/2 - inbound

I changed the automatic backup configuration cron job to save the file every hour so I can test it and see if it is working (only for test purposes).

Backup configuration of the External Router being saved automatically.