Applied Studies II

Logbook

Yuri Silva

Task: 3080 – Troubleshoot VLAN 133 - No connectivity for the Lamp server

Location: E1019

Date: March 18th

Time: 09:30AM – 12:30AM

Time Billed = 3 hours @ $75.00/hr = $225.00

- For this task the LAMP server was not able to ping its default gateway 192.168.133.1.

- I checked the IP configuration of the lamp server itself, and it was correct.

- I tested putting the LAMP in a different network (.143) to see if there was a problem with the VM. There was no problem with the VM itself.

- I checked the configuration on the Internal switch 2. The configuration was correct.

- I checked the configuration on the External router. The ACL to permit the 133 network was missing. So, I added that.

- It was still not working. There was no ping from the Lamp server to its default gateway.

- I checked the configuration on the Internal SW1 to see how the VLANs were.

- I found that the External router port g0/2 was connected to Internal SW port 27.

- I checked the VLAN configuration, and it was missing switchport configuration for an access port. Also adding VLAN 133.

- I then added the VLAN 133 to the proper port and configured as an access port.

- Tested again and this time it worked.

- The LAMP server 192.168.133.2 was able to ping the external router 192.168.133.1.

Task: 3081 – Reconfigure and troubleshoot NAT

Location: E1019

Date: March 19th

Time: 1:30PM – 4:30PM

Time Billed = 3 hours @ $75.00/hr = $225.00

- For this task NAT was supposed to have a new configuration which is to add a static NAT for the LAMP server.

- The configuration was created but it was still not working.

- So, I realized a new entry for the ACL needed to be added to allow traffic for the 133 network.

- In summary, the NAT would be:

  - g0/0 (207 network) – NAT outside.

  - g0/1 (153 network) – NAT inside

  - g0/2 (133 network) – NAT inside

- Another thing that had to be added was the port number for the NAT. Initially it was not added so it was not working properly. After the port number 80 (HTTP) was added, it started to work properly.

- Note: We kept the NAT overload configuration for the 143 network.

- Summary, now the external Router has the address 10.207.155.23 translated to the 192.168.143.

Task: 3082 – Installing Radius Server on DC1.

Location: E1019

Date: March 20th

Time: 9:00AM – 11:00AM

Time Billed = 2 hours @ $75.00/hr = $150.00

- The Radius server was installed based on the instructions provided for the AppStudies2 course.

- The idea is that users trying to connect to the AP could user their username and password registered on the AD of the domain.

- The Radius server was configured with MS-CHAP v2.

- Tyler is the one responsible to configure the AP so he provided me with the IP address of the AP (192.168.143.8).

- The service is running now.

- The shared key chosen was ILikeSurfing2!

- The group added was Domain users.

Task: 3083 –Troubleshooting DNS and the access to the t3webpage.

Location: E1019

Date: March 20th

Time: 11:00AM – 12:00PM

Time Billed = 1 hours @ $75.00/hr = $75.00

- The name resolution for the webpage was not working properly.

- The ping was successful when it was sent from the External router to the IP Address of web server 10.155.207.23 (address set up by static NAT).

- However, it was not working when it was sent to www.t3webpage.com

- The problem was that the DNS Server 192.168.143.5 was not set up on the router also the domain name look up was turned off.

- By updating this configuration the ping was successful.

- So the website is reachable from the 207 network by looking for its webpage name.

Bill for this week.

Pictures:

Radius Server MS-CHAP v2

RADIUS Clients added

Pinging the LAMP server webpage from the External Router (207 network).

Pinging Google from the External router.

External router running configuration

T3RtrE1019-1#sh run

Building configuration...

Current configuration : 3659 bytes

!

! Last configuration change at 10:08:52 AST Fri Mar 21 2025 by team3

! NVRAM config last updated at 10:20:51 AST Thu Mar 20 2025 by team3

! NVRAM config last updated at 10:20:51 AST Thu Mar 20 2025 by team3

version 15.2

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname T3RtrE1019-1

!

boot-start-marker

boot-end-marker

!

!

enable secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

!

no aaa new-model

clock timezone AST -4 0

!

ip cef

!

!

!

!

!

!

ip domain name team3.com

ip name-server 192.168.143.5

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

voice-card 0

!

!

!

!

!

!

!

!

license udi pid CISCO2911/K9 sn FGL171810P2

license boot module c2900 technology-package securityk9

hw-module pvdm 0/0

!

!

!

file prompt quiet

username pmcdevitt privilege 15 secret 4 78QJo8ceQjSnt34EQ1L2HK960cSqut1PgqNqk31V/62

username team3 privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username ysilva privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username engetchou privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username tarseneau privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username airanezereza privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

!

redundancy

!

!

!

!

!

ip ssh version 2

!

!

!

!

!

!

!

!

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

description External connection to the .207 Vlan.

ip address 10.155.207.41 255.255.255.0

ip nat outside

ip virtual-reassembly in

standby version 2

standby 1 ip 10.155.207.13

standby 1 priority 150

standby 1 preempt

duplex auto

speed auto

!

interface GigabitEthernet0/1

description Internal Gap connection to the 153 Vlan.

ip address 192.168.153.5 255.255.255.248

ip nat inside

ip virtual-reassembly in

standby version 2

standby 2 ip 192.168.153.1

standby 2 priority 150

standby 2 preempt

duplex auto

speed auto

!

interface GigabitEthernet0/2

description DMZ connection to the 133 Vlan

ip address 192.168.133.5 255.255.255.248

ip nat inside

ip virtual-reassembly in

standby version 2

standby 3 ip 192.168.133.1

standby 3 priority 150

standby 3 preempt

duplex auto

speed auto

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip nat inside source list 1 interface GigabitEthernet0/0 overload

ip nat inside source static tcp 192.168.133.2 80 10.155.207.23 80 extendable

ip nat inside source static 192.168.133.2 10.155.207.23

ip route 0.0.0.0 0.0.0.0 10.155.207.1

ip route 192.168.143.0 255.255.255.0 192.168.153.2

ip route 192.168.161.0 255.255.255.0 192.168.153.2

!

logging host 192.168.143.63

access-list 1 permit 192.168.143.0 0.0.0.255

access-list 1 permit 192.168.133.0 0.0.0.255

access-list 1 permit 10.155.207.0 0.0.0.255

!

!

!

control-plane

!

!

!

!

!

!

!

mgcp profile default

!

!

!

!

!

gatekeeper

shutdown

!

!

banner motd ^CAuthorized access Only^C

!

line con 0

password 7 00341215174C04140B711D

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport input all

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

privilege level 15

login local

transport input ssh

line vty 5 15

privilege level 15

login local

transport input ssh

!

scheduler allocate 20000 1000

ntp master 1

ntp server time.google.com

ntp server time.google.com

event manager applet Backup_Config

event timer cron cron-entry "30 1 * * 3"

action 1.0 cli command "enable"

action 2.0 cli command "copy running-config tftp://192.168.143.63/ExtRTR-1.cfg"

!

end