Applied Studies II

Logbook

Yuri Silva

Task: 3092 – Creating new security group for shared folder in between different teams

Location: E1019

Date: April 1st

Time: 10:00AM – 11:30AM

Time Billed = 1.5 hours @ $75.00/hr = $112.50

- Created a new OU called Common Group

- Created new security group called Common Group Global

- Set the group scope as Global

- Added 5 members from the sales teams to the group

- Spoke with Parwinder (from Team 1) to create a shared folder and add access to my global security group.

- Signed in a workstation as a user belonging to the global security group to check if the shared folder was showing up.

- It was not showing up yet.

- Will need to troubleshoot with Parwinder to figure out what the problem might be.

Task: 3093 – Troubleshooting Linux and Windows boxes for network connectivity

Location: E1019

Date: April 2nd

Time: 1:30PM – 3:00PM

Time Billed = 1.5 hours @ $75.00/hr = $112.50

- There was no connectivity from neither the Windows nor the Linux box.

- Verified all the configuration of the workstations, IP addresses, default gateway addresses and it was all correct

- Tried to ping the default gateway 192.168.143.1 from the Linux box and there was no success

- Tried to ping the windows box from the Linux box and the connection was successful.

- Investigated if there was a problem with the cable or the connection by using my own computer connected to the wall ethernet jack. Tried to ping default gateway and no success, I was able to ping the Windows box though.

- Realized the problem might be with the Switch configuration.

- Accessed the Internal Switch 2 and verified the VLAN Configuration.

- The ports g1/0/39 and g1/0/40 (used for both windows and linux boxes) were set up as trunk ports and they were allowing all vlans.

- Changed the configuration on those ports to make them access ports and assigned vlan 143 to them.

- Tried to ping again from the Linux to the default gateway and it was successful.

- Tried to ping again from the Windows to the default gateway and it was also successful.

- Both workstation had access to the Internet as well.

- Troubleshooting was completed.

Bill for this week.

Pictures:

New OU created called Common Group and new Global Security group for shared folder in between teams.

Members of the common global security group