Tyler Arseneau-Crowley

Applied Studies II

Logbook

Peter McDevitt

March 11 - 14

Task: 3072

Location: C1002

Date: March 11th, 2025

Time: 9:00 AM – 12:00 PM

Time Billed = 3 @ $75.00/hr = $225.00

We found out that there weren’t enough fiber drops in room C1002, so we could only use one of the new MLS switches, but we found out that the switch already in place is a layer 3 switch, so we added the configurations we did on to that switch and set up HSRP on all the team’s routers in C1002.

Task: 3073

Location: C1002

Date: March 12th, 2025

Time: 1:45 PM – 2:00 PM

Time Billed = 0.25 @ $75.00/hr = $18.75

We tested the failover by sending a continuous ping from my team's DC1 to the external router and then powering off my team's internal router. We know it worked because when the router was turned off, there was one timeout, and then it responded again, confirming that the failover was working as intended.

Task:3074

Location: E1019

Date: March 12th, 2025

Time: 2:15 PM – 3:30 PM

Time Billed = 1.25 @ $75.00/hr = $93.75

I was tasked to configure our wireless access point. To power the access point, I had to get a switch from room C2001 that had power over ethernet capabilities and then plug it into a port on the switch, then plug my laptop into a port, then turned of the wifi to my laptop and set my laptops IP to one that was in the same network as my access point which was (192.168.100.22). I was able to access the web GUI by typing in the IP of the access point and attempt to configure the access point.

| Task ID | Date | Start Time | End Time | Hours | Rate | Costs |
| --- | --- | --- | --- | --- | --- | --- |
| 3072 | March 11th 2025 | 9:00 AM | 12:00 PM | 3 | $225.00 | $225.00 |
| 3073 | March 12th 2025 | 1:45 PM | 2:00 PM | 0.25 | $18.75 | $18.75 |
| 3074 | March 12th 2025 | 2:15 PM | 3:30 PM | 1.25 | $93.75 | $93.75 |
|  |  |  | Totals: | 4.50 |  | 337.50 |
|  |  |  | Total After Taxes (15%) |  |  | $388.13 |