Tyler Arseneau-Crowley

Applied Studies II

Logbook

Peter McDevitt

March 18 - 21

Task: 3076

Location: Cybersecurity Storage Room

Date: March 18th, 2025

Time: 9:00 AM – 9:30 AM

Time Billed = 0.5 @ $75.00/hr = $37.50

The POE switches that we were previously using to power our access point were not giving sufficient power to the access point, so we replaced them with a POE+ switch that we got from the cybersecurity storage room.

Task: 3077

Location: E1019

Date: March 18th, 2025

Time: 9:30 AM – 10:30 AM

Time Billed = 1 @ $75.00/hr = $75.00

I configured the AP using the cli to the specifications provided. I disabled the 2.4GHz radio, made the AP run at half power, gave the AP the name T3AP and SSID of T3AppStd, gave the AP the ip address of 192.168.143.8, configured SSH, disabled dhcp and enabled logging to our syslog server. The only thing missing is to test our radius server with our AP.

Task:3078

Location: E1019

Date: March 19th, 2025

Time: 1:45 PM – 2:00 PM

Time Billed = 0.25 @ $75.00/hr = $18.75

I was tasked to configure the new POE+ switch that will be powering all the team's APs through the wall ports in the classroom. All I had to do was copy the configuration from the POE switch that we were previously using which was allowing the teams .14x networks to access the ports that the teams were connected to and trunking the 24th port.

Task:3079

Location: E1019

Date: March 19th, 2025

Time: 2:15 PM – 3:30 PM

Time Billed = 1.25 @ $75.00/hr = $93.75

I verified that the configuration from the previous day was correct and researched how to get the ap to show up for a device to connect to which was enabling guest mode on the 5GHz radio

| Task ID | Date | Start Time | End Time | Hours | Rate | Costs |
| --- | --- | --- | --- | --- | --- | --- |
| 3076 | March 18th 2025 | 9:00 AM | 9:30 PM | 0.5 | $37.50 | $37.50 |
| 3077 | March 18th 2025 | 9:30 AM | 10:30 AM | 1 | $75.00 | $75.00 |
| 3078 | March 19th 2025 | 1:45 PM | 2:00 PM | 0.25 | $18.75 | $18.75 |
| 3079 | March 19th 2025 | 2:15 PM | 3:30 PM | 1.25 | $93.75 | 93.75 |
|  |  |  | Totals: | 3 |  | $225.00 |
|  |  |  | Total After Taxes (15%) |  |  | $258.75 |