Tyler Arseneau-Crowley

Applied Studies II

Logbook

Peter McDevitt

January 14 – 17

Task: Cyclades

Location: C1002

Date: January 14th

Time: 9:45AM - 10:30AM

Time Billed = .75 hours @ $75.00/hr = $56.25

I was assigned to check on the Cyclades box in room C1002. The boss had to log in so I was able to check on the Cyclades box and when trying to ping 8.8.8.8 it timed out. So I pluged my own laptop into the switch the Cyclades box was connected to and changed my IP settings to 10.155.204.252/24 with a gateway of 10.155.204.1 and dns of 8.8.8.8 and changed pinging 8.8.8.8 and it also timed out. I was not able to fix it on my own.

Task: 3002 & 3004

Location: C1002

Date: January 14th

Time: 10:45AM – 12:00PM

Time Billed = 1.25 hours @ $75.00/hr = $93.75

I was assigned to configure the router and switch in room C1002. I first consoled into the switch and made the Vlan’s required which were 143, 153 and 161. Then, I assigned the correct ports to those Vlan’s which were ports 3,11 and 19. I then consoled into the router and configured the hostname as T3IntRtr. Then configured G0/0 with the IP address of 192.168.143.1 and subnet mask of /24 with a description of “Internal connection for the domain on the 143 Vlan.”. Configured G0/1 with the IP of 192.168.152.2 and subnet mask of /30 with a description of “Internal Gap connection to the 153 Vlan.”. Configured G0/2 with the IP address of 192.168.161.3 and subnet mask of /29 with a description of “Backdoor connection”. I then got a green ethernet cable and connected port 3 on the switch with G0/0 on the router. Then a purple ethernet cable connected port 11 with G0/1. Then an orange ethernet cable connected port 19 with G0/2.

| Task ID | Date | Start Time | End Time | Hours | Rate | Costs |
| --- | --- | --- | --- | --- | --- | --- |
| Cyclades | January 14th 2025 | 9:45AM | 10:30AM | .75 | $75.00 | $56.25 |
| 3 & 5 | January 14th 2025 | 10:45AM | 12:00PM | 1.25 | $75,00 | $93.75 |
|  |  |  | Totals: | 2 |  | $150.00 |
|  |  |  | Total After Taxes (15%) |  |  | $172.50 |

Name: Elianne Ngetchou
 Team: Team 3

Logbook for the Week

Task #307

Location: Office (E-1017)
 Date: 2025/01/14
 Time: 8:30 AM – 10:00 AM
 Time Billed: 1.5 hours * $75/hr = $112.50

Description:
 I was assigned to obtain a Dell OptiPlex mini laptop to install Ubuntu 2.4 and perform basic configurations.

- First, I downloaded the ISO file for Ubuntu. Then, I verified if the laptop was compatible with the downloaded version of Ubuntu.

- Next, I copied the ISO file to a USB drive, connected the mini laptop to a monitor, and inserted the USB drive.

- On the Ventoy page, I selected the ISO file to be used, chose the USB as the target, and started the process to make the USB bootable.

- Afterward, I restarted the laptop and pressed the "Esc" key to enter the BIOS/UEFI settings to set the USB as the first boot device. I saved the changes and exited.

- During the installation setup, I followed the on-screen instructions.

- Finally, after the installation was complete, I configured the IP address and username statically in the settings.

Task #2a

Location: Office (E-1017)
 Date: 2025/01/14
 Time: 10:30 AM – 12:30 PM
 Time Billed: 2 hours * $75/hr = $150

Description:
 I was tasked with installing Windows Server 2022 on a server in Office E-1017. However, I encountered issues during the process.

- The installation failed due to unknown problems.

- After collaborating with other team members, we identified two main issues:

  - The RAID was not functioning properly.

  - When the RAID worked, the Windows Server 2022 installation kept returning an error message.

Task #2b

Location: Office (E-1017)
 Date: 2025/01/15
 Time: 1:30 PM – 3:30 PM
 Time Billed: 2 hours * $75/hr = $150

Description:
 After conducting research the previous day, we identified several theories. The solutions that worked were as follows:

- Removing the SD card from the server so it could boot directly from the disk.

- During the customization process of the disk during Windows installation, we deleted all the deletable partitions.

After implementing these solutions, the installation was successful. However, I did not have time to configure the server afterward.

| Team member | Task ID |  | Task Description | Date | Start Time | End Time | Work Description | Total Hours | Billable Amount |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Elianne Ngetchou | 2a |  | Windows Server 2022 Installation on the server | Jan 14th | 10:30pm | 12:30pm | I was tasked with installing Windows Server 2022 on the Team 3 server in Office E-1017 but encountered issues during the process. The installation failed | 2 hours | $150.00 |
| Elianne Ngetchou | 2b | Finding a Solution to Windows Server 2022 Failed Installation |  | Jan 15th | 1:30pm | 2:30pm | Continuing from the previous workday, we identified the root cause of the issue and removed all existing partitions from the disk and SD card. After clearing and initializing the disk to the GPT format, the installation was successfully completed. | 2 hours | $150.00 |
| Elianne Ngetchou | 1 |  | Installing Linux (Ubuntu 2.4) and Configuring Network Settings | Jan 14th | 8:30am | 10:00am | I installed Ubuntu 2.4 the team PC and configured its network settings. | 1.5hours | $112.50 |

Total: 112.5+150+ (262.5*0.15) = 301.875