Applied Studies II

Logbook

Yuri Silva

Task: 3029 – Configure ntp server on external router

Location: E1019

Date: February 04th

Time: 09:00AM – 10:00AM

Time Billed = 1 hours @ $75.00/hr = $75.00

- Important to mention that I needed to fix the IP routing table to include the next hop as an IP address instead of an interface. Now the router can successfully ping 8.8.8.8. and has access to the Internet.

- Next step was to set up the DNS server of the router to google dns server 8.8.8.8.

- The router has a DNS server now and it can understand the name of the ntp server that will be added.

- Next step was to point to time.google.com as the ntp server

- Finally, time is correctly synchronized now both router and computer have the same time.

Task: 3030 – Configure new routing table on internal router

Location: E1019

Date: February 04th

Time: 09:00AM – 10:00AM

Time Billed = 1.5 hours @ $75.00/hr = $75.00

- Two new entries were added to the routing table on the internal router.

- The idea is so that linux and windows clients on our network can reach team1 and team5 address in their network. But not for team2 and team4 networks.

- So, the new routes created were:

ip route 192.168.141.0 255.255.255.0 192.168.161.1

ip route 192.168.145.0 255.255.255.0 192.168.161.5

- The concept is that whenever a packet needs to be sent to 192.168.141.x network, the next hop will the backdoor network 192.168.161.1

- The same concept is used for 192.168.145.x network. It will be sent to 192.168.161.5.

Task: 3039 – Configure ACLs on Internal and External Router

Location: E1019

Date: February 05th

Time: 13:30AM – 15:00AM

Time Billed = 1.5 hours @ $75.00/hr = $75.00

- The goal is to block traffic from Team 2 and Team 4.

- To do that the ACLs will be applied on both internal and external routers.

- The objective is to block traffic coming from host 192.168.161.2 and 192.168.161.4 on interface G0/2 of internal router. Which is the access from the back door internal network. Inbound traffic.

- Also to block access from 207 network. So that would be interface g0/2 of external routers and ip addresses 10.155.207.12 and 10.155.207.14. Inbound traffic as well.

- So the commands to create the ACLs are:

- Internal Router

access-list 10 deny host 192.168.161.2

access-list 10 deny host 192.168.161.4

access-list 10 permit any

int g0/2

ip access-group 10 in

- External Router

access-list 10 deny host 10.155.207.12

access-list 10 deny host 10.155.207.14

access-list 10 permit any

int g0/2

ip access-group 10 in

Bill for this week.

Pictures:

Configuring ntp server on external router.

Important to mention that I needed to fix the ip route table to include the next hop as an IP address instead of an interface. This is the updated routing table.

The DNS server for the external router was set up as google 8.8.8.8.

Time is correctly synchronized now both router and computer have the same time.

External router as a ntp master as well

Configured new routing table on internal Router