Applied Studies II

Logbook

Yuri Silva

Task: 3054 – Configure new External Router and HSRP

Location: E1019

Date: February 18th

Time: 09:30AM – 12:30AM

Time Billed = 3 hours @ $75.00/hr = $75.00

- The new router was taken from C2001 and brought to E1019.

- I copied the Backup running configuration of the External Router from the server (10.155.207.76) to my personal computer.

- I configured just one port of the External Router 2 with an IP address and configured my personal computer NIC to be in the same network.

- Then I created an ftp server on my personal computer.

- I copied the running configuration from the ftp server to the External Router 2.

- I changed the physical ip addresses of both External Router 1 and External Router 2.

- For the networks 207 and 153.

- HSRP was configured on both routers. External Router 1 with higher priority (150) so it would become the active router.

- Then I checked the configuration with the command show standby.

- The configuration worked properly.

Task: 3054 – Draw Network Diagram and AD Diagram

Date: February 12th

Time: 13:30AM – 5:30PM

Time Billed = 4 hours @ $75.00/hr = $75.00

- First, I drew a sketch on a piece of paper of the whole network topology.

- I discussed with my team to get information about the new infrastructure that was added to the project.

- I documented all VMs ip addresses and where they were installed. For both servers of Team3 (ours) and from Team5. Also documented the new AD (domains, groups, OUs, and users).

- Next, I installed Visio and added the Network Stencils.

- I drew the diagram on Visio to reflect the network diagram I had already drawn in a piece of paper.

- Finally, I drew the AD Diagram to add the domains information.

Bill for this week.

Pictures:

HSRP set up on both External routers.

Current running config on External router 1

T3RtrE1019-1#sh run

Building configuration...

Current configuration : 3333 bytes

!

! Last configuration change at 10:33:47 AST Wed Feb 19 2025 by team3

! NVRAM config last updated at 11:44:18 AST Tue Feb 18 2025 by team3

! NVRAM config last updated at 11:44:18 AST Tue Feb 18 2025 by team3

version 15.2

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname T3RtrE1019-1

!

boot-start-marker

boot-end-marker

!

!

enable secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

!

no aaa new-model

clock timezone AST -4 0

!

ip cef

!

!

!

!

!

!

ip domain name team3.com

ip name-server 8.8.8.8

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

voice-card 0

!

!

!

!

!

!

!

!

license udi pid CISCO2911/K9 sn FGL171810P2

license boot module c2900 technology-package securityk9

hw-module pvdm 0/0

!

!

!

file prompt quiet

username pmcdevitt privilege 15 secret 4 78QJo8ceQjSnt34EQ1L2HK960cSqut1PgqNqk31V/62

username team3 privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username ysilva privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username engetchou privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username tarseneau privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username airanezereza privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

!

redundancy

!

!

!

!

!

ip ssh version 2

!

!

!

!

!

!

!

!

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

description External connection to the .207 Vlan.

ip address 10.155.207.41 255.255.255.0

ip nat outside

ip virtual-reassembly in

standby version 2

standby 1 ip 10.155.207.13

standby 1 priority 150

standby 1 preempt

duplex auto

speed auto

!

interface GigabitEthernet0/1

description Internal Gap connection to the 153 Vlan.

ip address 192.168.153.5 255.255.255.248

ip nat inside

ip virtual-reassembly in

standby version 2

standby 2 ip 192.168.153.1

standby 2 priority 150

standby 2 preempt

duplex auto

speed auto

!

interface GigabitEthernet0/2

description DMZ connection to the 133 Vlan

ip address 192.168.133.5 255.255.255.248

standby version 2

standby 3 ip 192.168.133.1

standby 3 priority 150

standby 3 preempt

duplex auto

speed auto

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip nat inside source list 1 interface GigabitEthernet0/0 overload

ip route 0.0.0.0 0.0.0.0 10.155.207.1

ip route 192.168.143.0 255.255.255.0 192.168.153.2

ip route 192.168.161.0 255.255.255.0 192.168.153.2

!

access-list 1 permit 192.168.143.0 0.0.0.255

!

!

!

control-plane

!

!

!

!

!

!

!

mgcp profile default

!

!

!

!

!

gatekeeper

shutdown

!

!

banner motd ^CAuthorized access Only^C

!

line con 0

password 7 00341215174C04140B711D

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport input all

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

privilege level 15

login local

transport input ssh

line vty 5 15

privilege level 15

login local

transport input ssh

!

scheduler allocate 20000 1000

ntp master 1

ntp server time.google.com

event manager applet Backup_Config

event timer cron cron-entry "0 * * * *"

action 1.0 cli command "enable"

action 2.0 cli command "copy running-config tftp://10.155.207.76/ExtRTR-1.cfg"

!

End

Running config on External router 2

T3RtrE1019-2#sh run

Building configuration...

Current configuration : 3116 bytes

!

! Last configuration change at 14:11:18 AST Tue Feb 18 2025 by team3

version 15.2

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname T3RtrE1019-2

!

boot-start-marker

boot-end-marker

!

!

! card type command needed for slot/vwic-slot 0/0

enable secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

!

no aaa new-model

clock timezone AST -4 0

!

ip cef

!

!

!

!

!

!

ip domain name team3.com

ip name-server 8.8.8.8

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

voice-card 0

!

!

!

!

!

!

!

!

license udi pid CISCO2911/K9 sn FGL1832112Z

license boot module c2900 technology-package securityk9

hw-module pvdm 0/0

!

!

!

file prompt quiet

username pmcdevitt privilege 15 secret 4 78QJo8ceQjSnt34EQ1L2HK960cSqut1PgqNqk31V/62

username team3 privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username ysilva privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username engetchou privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username tarseneau privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

username airanezereza privilege 15 secret 4 z9e.ISmQWRybWB9UYvJ5NK8PkJAXSWdB7g0iZNI.hAI

!

redundancy

!

!

!

!

!

ip ssh version 2

!

!

!

!

!

!

!

!

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

description External connection to the .207 Vlan.

ip address 10.155.207.42 255.255.255.0

ip nat outside

ip virtual-reassembly in

standby version 2

standby 1 ip 10.155.207.13

duplex auto

speed auto

!

interface GigabitEthernet0/1

description Internal Gap connection to the 153 Vlan.

ip address 192.168.153.6 255.255.255.248

ip nat inside

ip virtual-reassembly in

standby version 2

standby 2 ip 192.168.153.1

duplex auto

speed auto

!

interface GigabitEthernet0/2

description DMZ connection to the 133 Vlan

ip address 192.168.133.6 255.255.255.248

standby version 2

standby 3 ip 192.168.133.1

duplex auto

speed auto

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip nat inside source list 1 interface GigabitEthernet0/0 overload

ip route 0.0.0.0 0.0.0.0 10.155.207.1

ip route 192.168.143.0 255.255.255.0 192.168.153.2

ip route 192.168.161.0 255.255.255.0 192.168.153.2

!

access-list 1 permit 192.168.143.0 0.0.0.255

!

!

!

control-plane

!

!

!

!

!

!

!

mgcp profile default

!

!

!

!

!

gatekeeper

shutdown

!

!

banner motd ^CAuthorized access Only^C

!

line con 0

password 7 00341215174C04140B711D

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport input all

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

privilege level 15

login local

transport input ssh

line vty 5 15

privilege level 15

login local

transport input ssh

!

scheduler allocate 20000 1000

ntp master 1

ntp server time.google.com

event manager applet Backup_Config

event timer cron cron-entry "0 * * * *"

action 1.0 cli command "enable"

action 2.0 cli command "copy running-config tftp://10.155.207.76/ExtRTR-2.cfg"

!

end